## Computer Architecture: RISC-V Assignment

### Instructions

- Topic: RV32I Instruction Set, Encoding, and C-to-Assembly Translation

- Time Allotted: 1 Week

- Tool(optional): VENUS RISC-V simulator.

- Submission: Please enter your name google id and roll number.

- Marks: 50

Roll Number: 23007

NAME: Fatima Shahid

Google ID: shahid4602264@cloud.neduet.edu.pk

### Questions

- Compare and contrast the immediate value encoding in I-type, S-type, and B-type instructions in RV32I. Discuss the rationale behind the different ways the immediate values are structured and sign-extended in these formats. How do these encoding choices impact the range of values that can be represented? (3 marks)

Answer:

### Comparison of Immediate Value Encoding in RV32I Formats

| Format | Immediate Bits | Structure | Range (after sign-extension) |
| --- | --- | --- | --- |
| I-type | 12 (bits [31:20]) | Contiguous | −2048 to +2047 |
| S-type | 12 (split [31:25], [11:7]) | Split across fields | −2048 to +2047 |
| B-type | 12 (shuffled, imm[0]=0) | Split & shifted left by 1 | −4096 to +4094 |

### Rationale Behind Immediate Encoding in I, S, and B Formats

I-type: Immediate is kept contiguous (bits [31:20]) since only one source register is needed, leaving space for a clean 12-bit field → simple decoding.

S-type: Needs two registers (rs1, rs2), so the 12-bit immediate is split across unused fields ([31:25] and [11:7]) to fit everything in 32 bits maintaining the simplicity of RISC-V.

B-type: Branches need larger range and alignment, so the immediate is shuffled and shifted left by 1, saving a bit while allowing ±4 KB jump range.

- What is the purpose of the zero register (x0) in the RISC-V architecture? Explain why having a hardwired zero register is beneficial for assembly language programming. (2 marks)

Answer:

The zero register (x0) in RISC-V is hardwired to the constant 0, meaning it always reads as 0 and discards any value written to it.

Benefit: It simplifies assembly programming by eliminating the need for a dedicated instruction to load zero, enabling simpler code (e.g., comparisons, clearing registers, immediate arithmetic) and reducing hardware complexity.

- Translate the following C code into RV32I assembly. Assume that the integer variables a, b, and c are stored in registers s0, s1, and s2 respectively. (5 marks)

C-code

a = b + 15;

c = a - 3;

Answer:

addi s0, s1, 15    # a = b + 15   (s0 = s1 + 15)

addi s2, s0, -3    # c = a - 3    (s2 = s0 - 3)

- Decode the following 32-bit RV32I instruction given in hexadecimal: 0x00858633. Show your work by identifying the instruction format, the opcode, and the values of the register and immediate fields. What operation does this instruction perform? (4 marks)

Answer:

- In binary : 00000000100001011000011000110011₂

- Format: R-type (because opcode = 0110011 = 0x33) ie: the last 7 bits

Field breakdown

R-type = [funct7 (7b) | rs2 (5b) | rs1 (5b) | funct3 (3b) | rd (5b) | opcode (7b) ]:

- funct7 (31–25): 0000000 → 0

- rs2 (24–20): 01000 → x8 (ABI: s0/fp)

- rs1 (19–15): 01011 → x11 (ABI: a1)

- funct3 (14–12): 000 → ADD/SUB group

- rd (11–7): 01100 → x12 (ABI: a2)

- opcode (6–0): 0110011 → R-type ALU

operation:

- opcode=0x33, funct3=000, funct7=0000000 ⇒ ADD

Final assembly:

- add x12, x11, x8
 (Application Binary Interface : add a2, a1, s0)

Operation performed:

- x12 = x11 + x8 (i.e., a2 = a1 + s0).

- Encode the following RISC-V instructions to machine language instructions (6 marks)

  - slli x9, x6, 15

  - sw x19, 1240(x21)

Answer:

Instruction : slli x9, x6, 15

Type: I-type (Shift immediate is a special case).

Fields:

  - opcode = 0010011 (OP-IMM)

  - funct3 = 001 (SLLI)

  - funct7 = 0000000 (for SLLI)

  - rd = x9 → 01001₂

  - rs1 = x6 → 00110₂

  - shamt = 15 → 01111₂

Binary Fields:

| imm[11:0] | rs1 | funct3 | rd | opcode |
| --- | --- | --- | --- | --- |
| 000000001111 | 00110 | 001 | 01001 | 0010011 |

Machine Language Instruction: 0x00F31493

Instruction : sw x19, 1240(x21)

Type:S-type

Fields:

  - opcode = 0100011 (store)

  - funct3 = 010 (SW)

  - rs1 = x21 → 10101₂

  - rs2 = x19 → 10011₂

  - imm = 1240 → decimal → binary = 010011011000₂ (12 bits)

- imm[11:5] = 0100110

- imm[4:0] =  11000

Binary Fields:

| Immediate [11:5] | rs2 | rs1 | funct3 | imm[4:0] | opcode |
| --- | --- | --- | --- | --- | --- |
| 0100110 | 10011 | 10101 | 010 | 11000 | 0100011 |

Machine Language Instruction: 0x4D3AAC23

- Write a complete RV32I assembly program that implements the following C code. The base address of the integer array my_array is stored in register s0. The sum should be stored in register s1. Your code should be well-commented to explain the logic of each instruction. (5 marks)

C-code

int my_array[] = {5, 2, 8, 4, 1};

int sum = 0;

for (int i = 0; i < 5; i++) {

sum += my_array[i];

}

Answer :

addi s1, x0, 0        # sum = 0

addi t0, x0, 0        # i = 0

addi t1, x0, 5         #bound set to 5

loop:

beq  t0, t1, end      # if i == 5, exit loop

slli t2, t0, 2        # offset = i * 4 (word = 4 bytes)

add  t2, s0, t2       # address = base + offset

lw   t1, 0(t2)        # load my_array[i] into t1

add  s1, s1, t1       # sum += my_array[i]

addi t0, t0, 1        # i++

jal  x0, loop         # jump back to loop

end:

# sum is stored in s1

# program ends here

- Translate the following C function, which includes an if-else statement, into an equivalent RV32I assembly program. Assume a is in a0 and b is in a1. The return value should be placed in a0. Explain how you used branch instructions to implement the conditional logic. (5 marks)

C-code

int conditional_logic(int a, int b) {

if (a > b) {

return a - b;

} else {

return b - a;

}

}

Answer:

# Arguments:

# a0 = a

# a1 = b

# Return value will be in a0

ble a0, a1, else_part    # if (a <= b) jump to else_part

# if-part: return a - b

sub a0, a0, a1           # a0 = a - b

return: jalr x0, 0(ra)                       # return

else_part:

sub a0, a1, a0           # a0 = b - a

return: jalr x0, 0(ra)                                    # return

- Assume that register x10 contains the base address of array1, and x11 corresponds to n (size of array1). Compile the following C function into RISC-V assembly language. Please archive all temporary/saved registers that you use inside your RISC-V code onto stack. (5 marks)

C-code

void clear1(int array1[], unsigned int n) {

unsigned int i;

for (i = 0; i < n; i += 1)

{

array1[i] = 0;

}

}

Answer:

# a0 = base address of array
	 # a1 = n (size)

Clear1:
 	addi sp, sp, -8   	# make space on stack
 	sw x5, 0(sp)      	# save x5
 	sw x6, 4(sp)      	# save x6
 	addi x5, x0, 0    	# i = 0

 Loop:

	 bge x5, a1, End_loop  # if (i >= n) break
 	slli t0, x5, 2    	# t0 = i * 4
 	add  t0, a0, t0 	# t0 = base + offset
 	sw   x0, 0(t0)    	# array[i] = 0
 	addi x5, x5, 1    	# i++
 	jal x0, loop      	# repeat

 End_loop:

 	lw x5, 0(sp)      	# restore x5
 	lw x6, 4(sp)      	# restore x6
 	addi sp, sp, 8
 	jalr x0, 0(ra)    	# return

- Design and implement a complete RV32I assembly function named find_max that finds the maximum value in an integer array. The function should take two arguments: the base address of the array (in register a0) and the number of elements in the array (in register a1). (10 marks)

The function must follow the RISC-V calling convention, which includes:

  - Setting up a stack frame.

  - Saving any callee-saved registers (s0-s11) that are used.

  - Placing the maximum value found in register a0 before returning.

  - Restoring the saved registers and tearing down the stack frame before returning using ret.
Provide the complete, well-documented assembly code for the find_max function.

  - Hint: Remember to allocate space on the stack for the return address (ra) and any s registers you use. The stack pointer (sp) must be adjusted at the beginning and end of your function.

.text

.globl find_max

Find_max:

# Prologue: create stack frame, save callee-saved registers

# We allocate 8 bytes:

addi    sp, sp, -8       # allocate 16 bytes on stack

sw      ra, 4(sp)        # save return address

sw      s0, 0(sp)         # save old frame pointer (callee-saved)

addi    s0, sp, 8        # set frame pointer (s0 = old sp)

################################################################

# Arguments on entry:

#   a0 = base address

#   a1 = number of elements (n)

# We'll use temporaries:

#   t0 = base pointer (unchanged)

#   t1 = n (length)

#   t2 = loop index i

#   t3 = loaded array element

# s0 will hold current max value

################################################################

# handle n <= 0 : return 0

blez    a1, .return_zero

# initialize

addi   t0, a0, 0            # t0 = base pointer

addi      t1, a1,0            # t1 = n

addi    t2, x0, 1              # t2 = i = 1  (we'll use arr[0] as initial max)

lw      s0, 0(t0)         # s0 = arr[0]  (initial maximum)

.loop:

bge     t2, t1, .done     # if i >= n, finished

slli    t3, t2, 2         # t3 = i * 4  (word offset)

add     t3, t0, t3        # t3 = address of arr[i]

lw      t3, 0(t3)         # t3 = arr[i]

blt     s0, t3, .update   # if arr[i] > current_max -> update

addi    t2, t2, 1         # i++

jal x0,  .loop

.update:

addi      s0, t3,0            # s0 = arr[i]  (new max)

addi    t2, t2, 1         # i++

jal x0,      .loop

.done:

addi      a0, s0,0            # move maximum into a0 for return

################################################################

# Epilogue: restore saved registers and tear down stack frame

################################################################

lw      ra, 4(sp)        # restore ra

lw      s0, 0(sp)         # restore old s0

addi    sp, sp, 8        # deallocate stack frame

jalr x0, 0(ra)

.return_zero:

li      a0, 0             # define behavior for empty array: return 0

lw      ra, 4(sp)        # restore ra

lw      s0, 0(sp)         # restore s0

addi    sp, sp, 8      # restore sp

jalr x0, 0(ra)